NI4 FACT SHEET

| Dimensions (metres). | Length | Width | Height |
| --- | --- | --- | --- |
| Stored | 14 | 2.5 | 4 |
| Operational | 14 | 4.25 | 4 |
| Weight | 33 T |  |  |

Power

1 x 125 Amp 3 Phase.

UPS Back system for 20 minutes, Media protection only.

VISION SYSTEM	ST 2110-20 Video over IP system with -07 Redundancy.

Vision Mixer

Sony MLS-X1 with 96 inputs and 64 Outputs at HD.

Vision System

22 x Imagine Communications, Selenio Network Processors IP Multiviewers.

14 x Imagine Communications, Selenio Network Processors IP HD Gateways (Internal) with 76 HD Inputs, 176 HD outputs each with 16 Channels embedded audio.

Cameras

System Cameras	13 x Sony HDC-3500 HD/3G, 4K and HFR by license, wired for 16.

Slomo Cameras		1 x Sony HDC-3500 HFR (x2) (x3) (x4) HD/3G, 4K by license.

Box Camera		1 x Sony P50 HD/3G, 4K and HFR by license.

PTZ Cameras		2 x BRC-800 Remote PTZ HD/3G; Wired for 4.

Radio Cameras 	2 x Videosys Systems; Wired for 4.

Optics

Canon studio lenses.

Canon EFP lenses.

On Board recording

2 x EVS XT-VIA 12 Channel with 277 Hours capacity each at DNxHD120 bit rate, wired for 4 units.

EVS -IP Director system with removable storage, wired for 2.

2 x Atomos Shogun Studio2, 8 Channel recorders at HD.

External

Wired for 14 dual redundant stage boxes on TAC4 optical fibre.

6 x Imagine Communications Selenio Network Processors IP HD Gateways on remote video stage boxes with 56 HD Inputs 24 HD outputs.

4 x Calrec Audio Ltd, stage boxes Audio Large size with 192 Inputs x 64 Outputs.

2 x Calrec Audio Ltd, stage boxes Audio Mid-size with 48 Inputs x 24 Outputs.

Audio System		ST 2110-30 Audio over IP system with -07 redundancy.

Audio Mixer

Calrec Audio Ltd, Artemis digital audio console, 48 fader digital Sound desk.

2 x Calrec Audio Ltd, Impulse IP Core with Bluefin3, 384 Channel with 4096 x 4096 Routes on dual redundant cores.

Communications System

Riedel 1024 IP Communication system with 148 x 148 ports and Riedel 1200 series smart panels.

2 x Riedel Commentators Panels.

Riedel Bolero Wireless talkback	with 13	Handsets.

2 x Motorola UHF Talkback with 12 handsets.

2 x Systembase Multiformat Codecs 4 channel (TBU's).

1 x Comrex 5 channel stereo IP Codec.

Audio Processing

1 x Waves Sound Grid Audio Processor 32 Channel.

Audio Record and Playback

1 x JoeCo audio recorder 64 channel Dante audio recorder.

1 x Reaper 128 channel, Dante and Madi, PC based audio recorder.

1 x SpotOn 16 channels PC based audio playback and record.

Graphics

2 x VizRt IP Graphics engine, wired for 4	.

Production Areas Seating

5 x Production front desk.

6 x Production rear desks.

3 x EVS.

3 x EDIT Area.

Production Monitoring

Vision			8 x OLED monitors with fully configurable Multiviewers. Up to 128 PIPs. With UMD and Red / Green Tally.

Sound 			5.1 Audio monitoring within production gallery with calibration system.

Network

Arista Spine / Leaf IP system with 7280CR3-96 series: 96 port QSFP100 - 9.6 Tbps wire speed performance with 16GB buffer - Supports 96 x 100G QSFP or up to 192 x 50G".

@3840 x 3840 Routes at 1080 / 50i a max number of streams 7680 in or out.

@1920 x1920 Routes at 1080 / 50p a max number of streams 3840 in or out.

1 x PepWave, Bonded 3/4/5G Mobile data connection.